DHCP Spoofing Attack

A DHCP spoofing attack occurs when an attacker sets up a rogue DHCP server within a network. This rogue server sends fraudulent DHCP responses to clients, tricking them into accepting incorrect IP configurations. The attacker can then redirect traffic through malicious gateways or DNS servers, enabling them to intercept data, launch man-in-the-middle attacks, or cause denial of service. The main objective is to manipulate network traffic for malicious purposes, compromising the network's security and integrity.

LAB on DHCP Spoofing Attack

Step for configurations

- Enter on Router and enable DHCP

R1(config)#int f0/0

R1(config-if)#ip address  10.0.0.1 255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#ip dhcp pool  DHCP

R1(dhcp-config)#network  10.0.0.0 255.0.0.0

R1(dhcp-config)#default-router  10.0.0.1

R1(dhcp-config)#dns-server  8.8.8.8

- Enter on All PC

PC> dhcp

- Launch	DHCP Attack

Enter on Kail

#

- Enter all PC for check ip

Secure of DHCP Spoofing

Enter on Switch

IOU1(config)#ip dhcp snooping  vlan  1

IOU1(config)#ip dhcp snooping

IOU1(config)#no ip dhcp snooping  information  option

IOU1(config)#interface  e0/0

IOU1(config-if)#no shutdown

IOU1(config-if)#ip dhcp  snooping  trust

IOU1(config)#interface  e0/3

IOU1(config-if)#no shutdown

IOU1(config-if)#ip dhcp snooping  limit  rate  3